🕐

ESP32-C3 Smart Clock

Full Technical Reference & Architecture Guide

Bug-Fixed & Heavily Annotated Edition

Written for electronics enthusiasts with a web development background.
Covers hardware wiring, firmware architecture, build tools, and every file in the project — from first principles.

# Table of Contents

1.  The Big Picture — What This Project Does	3

2.  Hardware — Components, Wiring & Electricity Basics	4

3.  The Embedded World vs the Web World	6

4.  Software Architecture — How the Code Is Organised	8

5.  Every File and Folder Explained	11

6.  The Build System — PlatformIO & the .pio Folder	13

7.  Every Build Command Explained	15

8.  Bug Fixes — What Was Wrong and Why	17

9.  OTA Updates — How Remote Firmware Updates Work	19

10.  WiFi Setup Flow — The Captive Portal	21

11.  The Display — Three Clock Faces & Rendering	22

12.  How to Customise — Your Roadmap to New Features	24

13.  Troubleshooting	25

# 1  The Big Picture — What This Project Does

This project turns a tiny computer chip (ESP32-C3) and a small screen (SSD1306 OLED) into a smart clock that:

- Connects to your home WiFi automatically.

- Fetches the exact current time from internet time servers (NTP).

- Shows three different clock face styles (digital, analog, minimal).

- Can update its own software wirelessly without plugging in a USB cable (OTA).

- Can be flashed for the first time directly from a web browser — no extra software needed.

## Technology Layers

Think of this project like a website, but for hardware. Each layer has a clear job:

| Layer | Web Equivalent | In This Project |
| --- | --- | --- |
| Hardware | The physical server/device | ESP32-C3 chip + SSD1306 OLED screen |
| Runtime / OS | Node.js / Browser engine | Arduino Framework (wraps ESP-IDF) |
| Language | JavaScript | C++ (with Arduino helpers) |
| Build tool | npm / webpack / vite | PlatformIO (pio) |
| Libraries | npm packages | Libraries in platformio.ini |
| Config | .env / config.js | include/config.h |
| Entry point | index.js / main.js | src/main.cpp |
| Deployment | npm run build + upload | ./build.sh build + flash |

💡 Key Insight If you can read JavaScript and understand how a React app breaks into components, you already understand the structural thinking behind this firmware. The concepts map almost 1:1 — only the language and environment differ.

# 2  Hardware — Components, Wiring & Electricity Basics

## The Two Components

| Component | What it is | Role in this project |
| --- | --- | --- |
| ESP32-C3 Super Mini | A microcontroller (tiny computer on a chip). RISC-V CPU @ 160 MHz, 4 MB flash, built-in WiFi & Bluetooth, USB-C port. | The "brain". Runs all the code, connects to WiFi, talks to the screen. |
| SSD1306 OLED 128×64 | A 0.96-inch display with 128 columns × 64 rows of individually controlled white pixels. | The "face". Shows the clock, WiFi status, and update progress. |

## Understanding I²C — How the Chip Talks to the Screen

The ESP32-C3 and OLED communicate over a protocol called I²C (pronounced "I-squared-C"). I²C needs only 2 wires to connect multiple devices:

| Wire | Name | Purpose |
| --- | --- | --- |
| SDA | Serial Data | Carries the actual data bytes back and forth (bidirectional). |
| SCL | Serial Clock | A timing signal. The "master" (ESP32) pulses this to say "now read a bit". |

Every device on the I²C bus has a unique address (like an IP address, but just a number 0–127). The SSD1306 OLED usually lives at address 0x3C. If the screen is blank, try 0x3D.

## Wiring Diagram

🔌 Wiring — Connect These 4 Wires Connect in order: power first, then ground, then data wires. Never connect/disconnect while powered.

| ESP32-C3 Pin | OLED Pin | Wire Colour (convention) | Function |
| --- | --- | --- | --- |
| 3.3V | VCC | Red | Power supply to the OLED (3.3 volts) |
| GND | GND | Black | Ground (0 volts — the reference for all measurements) |
| GPIO 20 | SDA | Blue/Green | I²C Data — defined as OLED_SDA in config.h |
| GPIO 21 | SCL | Yellow | I²C Clock — defined as OLED_SCL in config.h |

## The Boot Button (GPIO 9)

The ESP32-C3 Super Mini has one physical button already on the board — the BOOT button, connected to GPIO 9. In normal operation (after boot) the firmware uses it as a user interface button:

- Short press (< 2 seconds): Cycle through the three clock faces.

- Hold 2+ seconds: Erase saved WiFi credentials and reboot. Useful when you move to a new network.

The pin is configured with INPUT_PULLUP. This means: the pin reads HIGH (logic 1) when the button is NOT pressed, and LOW (logic 0) when it IS pressed. The internal resistor "pulls" the pin up to 3.3 V by default, and the button shorts it to 0 V when pressed. This is the standard, safe way to read a button without needing a physical resistor.

# 3  The Embedded World vs the Web World

Coming from a web / frontend background, some things will feel familiar and some will feel completely alien. This section bridges the gap.

## What Is a Microcontroller?

A microcontroller (MCU) is a complete computer on a single chip — CPU, RAM, and storage (flash) all in one package. It's designed to run one program, forever, directly on the hardware with no operating system in between.

| Aspect | Your Browser / Node.js | ESP32-C3 |
| --- | --- | --- |
| CPU Speed | 1–4 GHz | 160 MHz (25× slower) |
| RAM | 8–32 GB | 400 KB (20 million× less) |
| Storage | SSD (TBs) | 4 MB flash (1 million× less) |
| OS | Windows / macOS / Linux | None (bare metal + Arduino) |
| Input/Output | Mouse, keyboard, network | GPIO pins, I²C, SPI, WiFi |
| Power | ~100 W | ~0.5 W |
| Boot time | Minutes | ~1 second |
| Program start | User launches it | Powers on automatically, runs forever |

## The Arduino Framework — Your "Browser Engine" for Hardware

Writing code directly for the ESP32-C3 hardware (called "bare metal") is extremely difficult. The Arduino framework is an abstraction layer that gives you:

- setup() and loop(): Instead of an event loop, Arduino gives you two functions. setup() runs once, loop() runs forever.

- pinMode() / digitalWrite() / digitalRead(): Simple functions to control GPIO pins — like document.getElementById() but for physical hardware pins.

- Serial.println(): Prints to the USB serial port — exactly like console.log(), just viewed with a serial monitor instead of browser DevTools.

- delay(): Pauses execution. Like setTimeout() but blocking — the CPU does nothing else.

- Wire.begin(): Starts the I²C bus — like initialising a library in JavaScript.

## C++ vs JavaScript — Key Differences

This project is written in C++. Here's what a frontend developer needs to know:

| Concept | JavaScript | C++ (Arduino) |
| --- | --- | --- |
| Types | Loose (let x = 5) | Strict (int x = 5; uint32_t ms = millis();) |
| String | String, template literals | char[] arrays or Arduino String class |
| No GC | Garbage collected | Manual memory — prefer stack variables |
| Headers | import / require | #include "file.h" — declare before use |
| Classes | class MyClass {} | Declared in .h, implemented in .cpp |
| Null | null / undefined | nullptr for pointers, -1 or 0 for ints |
| Async | Promise, async/await | No OS threads — use millis() timing |
| Build | Browser / Node runs .js | Must compile to binary for the specific chip |

## millis() — The Embedded Alternative to setTimeout

There is no setTimeout() or async/await in embedded C++. Instead, everything is time-checked manually using millis() which returns how many milliseconds have passed since boot.

This pattern (called "non-blocking timing") is everywhere in this codebase:

// JavaScript equivalent: setInterval(() => redraw(), 500)

uint32_t lastRedraw = 0; // "timestamp of last run"

void loop() {

uint32_t now = millis(); // current time in ms since boot

if (now - lastRedraw >= 500) { // has 500ms passed?

lastRedraw = now; // reset the timer

redrawClock(); // do the work

}

// ... other non-blocking checks here

}

💡 Why not just use delay(500)? delay() freezes the CPU completely — nothing else runs. With millis() timing, the button checking, WiFi reconnection, and clock redrawing all happen "concurrently" in the same loop without blocking each other. This is the embedded equivalent of writing non-blocking async code.

# 4  Software Architecture — How the Code Is Organised

## Module Overview

The firmware is split into three modules (analogous to React components or Node.js modules). Each module has a header file (.h) that declares what it exposes, and a source file (.cpp) that implements it.

| Module | Files | Web Analogy | Responsibility |
| --- | --- | --- | --- |
| Main / Orchestrator | src/main.cpp | App.jsx / index.js | Wires everything together. Owns setup(), loop(), button handling, and screensaver. |
| Display Manager | include/display.h + src/display.cpp | A React UI component | All OLED output: 3 clock faces, boot screen, OTA progress bar, error messages. |
| Net Manager | include/net.h + src/net.cpp | An API / service layer | WiFi connection (captive portal), NTP time sync, OTA update download and flash. |

## Startup Sequence (setup())

The order matters. Each step depends on the previous one succeeding:

- Serial.begin(115200)   — Open the USB debug channel (like opening DevTools).

- display.begin()        — Initialise the OLED. If this fails, blink the LED and halt.

- display.showBootScreen()— Show the splash screen while hardware initialises.

- net.connect()          — Connect to WiFi via captive portal or saved credentials.

- display.showConnected()— Show the assigned IP address on screen.

- net.syncNTP()          — Fetch current time from internet time servers.

- net.checkOTA()         — Check if new firmware is available (downloads if so).

## Main Loop (loop())

Every iteration of loop() (roughly every 10ms) does these tasks in order:

void loop() {

handleButton(); // ~1ms: read GPIO9, detect press/release edges

net.loop(); // ~1ms: check WiFi, periodic OTA timer

// Screensaver: dim after 60 seconds of inactivity

if (now - lastActivity > SCREENSAVER_MS) display.setDimmed(true);

else display.setDimmed(false);

// Clock redraw every 500ms (CLOCK_UPDATE_MS)

if (now - lastClockUpdate >= CLOCK_UPDATE_MS) {

struct tm timeinfo;

if (getLocalTime(&timeinfo, 0)) display.update(&timeinfo); // draw clock

else if (connected) display.showMessage("Syncing...", NTP_SERVER1);

else display.update(&fallbackTime); // elapsed time

}

delay(10); // yield CPU to WiFi stack (prevents watchdog reboot)

}

## Inter-Module Communication

The three modules are loosely coupled. They communicate in two ways:

- Direct calls: main.cpp calls display.xxx() and net.xxx() methods directly through their class instances.

- extern function bridge: net.cpp needs to show OTA progress on the OLED, but importing display.h there would create a circular dependency. Instead, main.cpp defines a function displayOTAProgress(int pct) that calls display.showOTAProgress(). In net.cpp, the keyword extern declares "this function exists somewhere else, use it". The linker connects them at build time. Think of it like a callback.

// In main.cpp — defines the bridge

void displayOTAProgress(int pct) {

display.showOTAProgress(pct); // calls DisplayManager

}

// In net.cpp — declares "someone else defines this"

extern void displayOTAProgress(int pct);

// Inside net.cpp OTA download loop:

httpUpdate.onProgress([](int cur, int total) {

displayOTAProgress(cur * 100 / total); // calls main.cpp's bridge

});

# 5  Every File and Folder Explained

## Root Directory

| File / Folder | Purpose | Edit it? |
| --- | --- | --- |
| platformio.ini | The project's package.json. Defines board, framework, libraries, build flags, upload speed. | Yes — to add libraries or change board settings. |
| build.sh | Shell script with shortcuts: build, flash, monitor, clean, web. Run with ./build.sh <command> | Rarely — only to add new workflow commands. |
| include/ | Header files (.h). Declare classes, enums, and constants. Think of these as TypeScript interface files. | Yes — to add new config values or class methods. |
| src/ | Source files (.cpp). Implement everything declared in the headers. | Yes — main development area. |
| web/ | Static HTML + binary files for the browser-based web flasher. | Yes — after each build, copy new .bin files here. |
| .pio/ | Auto-generated by PlatformIO. Contains compiled binaries and downloaded libraries. Like node_modules/. | Never — auto-managed. |
| .gitignore | Tells git not to commit .pio/ (it's huge and regeneratable). | Rarely. |

## include/ — Header Files

| File | What It Declares |
| --- | --- |
| config.h | All hardware pins, WiFi AP name/password, NTP server URLs, OTA URL, timing constants. THE file to edit for customisation. |
| display.h | The DisplayManager class and ClockFace enum. Lists all public methods the class exposes. |
| net.h | The NetManager class. Lists connect(), syncNTP(), checkOTA(), loop() and state query methods. |

## src/ — Source Files

| File | Lines | What It Implements |
| --- | --- | --- |
| main.cpp | ~130 | setup(), loop(), handleButton(), displayOTAProgress() bridge. The app orchestrator. |
| display.cpp | ~300 | OLED drawing for all 3 clock faces, all status screens, brightness/dim control, and drawing helpers. |
| net.cpp | ~200 | WiFiManager captive portal, NTP sync retry loop, OTA version check, firmware download & flash. |

## web/ — Web Flasher Files

| File | Purpose |
| --- | --- |
| index.html | The browser UI. Contains the "Flash" button powered by the ESP Web Tools library (loaded from CDN). Also shows wiring instructions. |
| manifest.json | Tells ESP Web Tools where each binary goes in the ESP32 flash memory (at which byte offset). Like a deployment manifest. |
| version.json | Hosted on YOUR server. The clock fetches this to check if a new firmware is available. Contains version string and firmware download URL. |
| firmware.bin | The compiled application code. Copied here by ./build.sh web. |
| bootloader.bin | The ESP32 bootloader. Flashed once at offset 0x0000. Initialises hardware before jumping to the application. |
| partitions.bin | A map of how the 4 MB flash is divided between bootloader, OTA slots, NVS storage, etc. |
| boot_app0.bin | An Espressif-provided binary that manages which partition to boot from (supports A/B OTA switching). |

# 6  The Build System — PlatformIO & the .pio Folder

## What is PlatformIO?

PlatformIO is to embedded development what npm + webpack is to web development. It:

- Downloads and manages libraries (like npm install).

- Compiles your C++ source files into machine code for the specific chip (like webpack/vite bundling).

- Uploads the compiled binary to the device over USB (like deployment).

- Opens a serial monitor to see log output (like the browser console).

## platformio.ini — Annotated

This file is the equivalent of package.json. Here is every line explained:

[env:esp32-c3-supermini] ; The environment name. Used with -e flag.

platform = espressif32 ; Hardware platform: Espressif ESP32 family.

board = esp32-c3-devkitm-1 ; Closest official board profile to our Super Mini.

framework = arduino ; Use Arduino APIs (setup/loop, pinMode, etc).

; Override the board profile to match our specific hardware:

board_build.mcu = esp32c3 ; Force the RISC-V C3 core specifically.

board_build.f_cpu = 160000000L ; Run the CPU at 160 MHz (its maximum).

board_build.flash_mode = qio ; QIO = Quad I/O: uses 4 wires to read flash.

; Fastest mode. DIO (2 wire) is slower.

board_build.partitions = default.csv ; Use default partition layout.

monitor_speed = 115200 ; Baud rate for serial monitor (must match Serial.begin()).

monitor_filters = esp32_exception_decoder ; Decode crash stack traces automatically.

upload_speed = 921600 ; USB upload baud rate. 921600 = fastest reliable speed.

; Libraries (like package.json "dependencies"):

lib_deps =

adafruit/Adafruit SSD1306 @ ^2.5.9 ; OLED driver

adafruit/Adafruit GFX Library @ ^1.11.9 ; Drawing primitives (lines, circles, text)

adafruit/Adafruit BusIO @ ^1.15.0 ; I²C / SPI abstraction (Adafruit needs it)

bblanchon/ArduinoJson @ ^7.0.4 ; JSON parsing for OTA version.json

tzapu/WiFiManager @ ^2.0.17 ; Captive portal WiFi setup

; Compiler flags sent to the C++ compiler:

build_flags =

-D ARDUINO_USB_MODE=1 ; Enable native USB (ESP32-C3 has built-in USB)

-D ARDUINO_USB_CDC_ON_BOOT=1 ; Start USB CDC serial on boot (enables Serial.println)

-D CONFIG_ARDUHAL_LOG_DEFAULT_LEVEL=0 ; Silence internal ESP32 debug spam

## The .pio Folder — What's Inside

The .pio/ folder is auto-generated. Like node_modules/, never edit it and add it to .gitignore. Here's its structure:

| Path | Analogy | Contents |
| --- | --- | --- |
| .pio/libdeps/esp32-c3-supermini/ | node_modules/ | Downloaded library source code. Adafruit SSD1306, ArduinoJson, WiFiManager, etc. |
| .pio/build/esp32-c3-supermini/ | dist/ or build/ | Compiled .o object files and the final firmware.bin, bootloader.bin, partitions.bin. |
| .pio/build/esp32-c3-supermini/firmware.bin | dist/bundle.js | The final binary that gets flashed to the ESP32. |
| .pio/build/esp32-c3-supermini/firmware.elf | Source map | Debug symbols — used to decode crash addresses into function names. |
| .pio/build/esp32-c3-supermini/firmware.map | Bundle analyser output | Shows how much flash each function uses. |

## How Compilation Works

When you run pio run, this happens in order:

- Dependency resolution: Reads lib_deps from platformio.ini and downloads any missing libraries into .pio/libdeps/

- Preprocessing: Expands all #include and #define macros. config.h values get substituted directly into the code.

- Compilation: Each .cpp file is compiled independently into a .o (object) file — machine code for the ESP32-C3 RISC-V CPU.

- Linking: All .o files + library .a archive files are combined into one executable (firmware.elf). Function references (like extern displayOTAProgress) are resolved here.

- Binary extraction: The .elf is stripped down to raw bytes → firmware.bin. This is what gets flashed.

# 7  Every Build Command Explained

## Prerequisites — Install These Once

- Python 3.x — Required by PlatformIO. Install from python.org or your package manager.

- PlatformIO Core — The build tool. Install with:

pip install platformio

- VS Code + PlatformIO IDE extension — Optional but highly recommended. Gives you IntelliSense, build buttons, and a serial monitor GUI.

- Chrome or Edge — Required for the web flasher (Web Serial API is not supported in Firefox).

## ./build.sh Commands

The build.sh script is a convenience wrapper. You can also run the underlying pio commands directly — both are shown below.

### Build (Compile)

Compiles all source files and produces the firmware binary.

# Using the helper script:

./build.sh build

# What it actually runs:

pio run -e esp32-c3-supermini

# Output you will see:

# RAM: [==== ] 38.2% (used 15628 bytes from 40960 bytes)

# Flash: [===== ] 49.1% (used 515720 bytes from 1048576 bytes)

# ✅ Build done. Firmware: .pio/build/esp32-c3-supermini/firmware.bin

RAM% — How much of the 400KB working memory your code uses at runtime.

Flash% — How much of the 4MB permanent storage your compiled code occupies.

### Flash (Upload to Device)

Compiles (if needed) and uploads the firmware.bin to the connected ESP32-C3 via USB.

# Using the helper script:

./build.sh flash

# What it actually runs:

pio run -e esp32-c3-supermini -t upload

# If the device is not found, put it in bootloader mode:

# Hold the BOOT button, tap RESET, then release BOOT.

The -t upload flag tells pio to run the "upload" target after building. PlatformIO auto-detects the USB port (/dev/ttyUSB0 on Linux, COM3 on Windows, /dev/cu.usbmodem... on macOS).

### Monitor (Serial Console / Debug Output)

Opens a serial terminal to see Serial.println() output from the firmware. This is your DevTools console for embedded code.

# Using the helper script:

./build.sh monitor

# What it actually runs:

pio device monitor -e esp32-c3-supermini

# Example output you will see:

=== Smart Clock v1.0.0 ===

[NET] Starting WiFiManager...

[NET] Connected! IP: 192.168.1.42

[NET] Time synced: 14:32:07

[OTA] Already on latest firmware (v1.0.0)

[MAIN] Setup complete. Entering main loop.

# Press Ctrl+C to exit the monitor.

💡 Tip You can run build and monitor together: ./build.sh flash then immediately ./build.sh monitor. Or combine as one command: pio run -e esp32-c3-supermini -t upload && pio device monitor

### Clean (Reset Build Cache)

Deletes all compiled output. Forces a full recompile next time. Use this if you get strange build errors.

# Using the helper script:

./build.sh clean

# What it actually runs:

pio run -t clean

# Nuclear option: delete the entire .pio folder

rm -rf .pio/

# PlatformIO will re-download all libraries and recompile everything from scratch.

### Web (Copy Binaries for Web Flasher)

Copies the compiled binaries from .pio/build/ into the web/ folder, ready to serve.

# Using the helper script:

./build.sh web

# What it does:

cp .pio/build/esp32-c3-supermini/firmware.bin web/firmware.bin

cp .pio/build/esp32-c3-supermini/bootloader.bin web/bootloader.bin

cp .pio/build/esp32-c3-supermini/partitions.bin web/partitions.bin

# Also finds and copies boot_app0.bin from ~/.platformio/packages/

# Then serve the web flasher:

cd web && python3 -m http.server 8080

# Open http://localhost:8080 in Chrome or Edge

### All-in-One

Build, copy web files, and flash in one command:

./build.sh all

# Equivalent to:

./build.sh build && ./build.sh web && ./build.sh flash

## Other Useful pio Commands

| Command | What it does |
| --- | --- |
| pio pkg install | Reinstall all libraries from platformio.ini (like npm install after clone). |
| pio device list | List all connected USB devices and their port names. |
| pio run --list-targets | Show all available build targets for the project. |
| pio check | Run static analysis (find potential bugs before compiling). |

# 8  Bug Fixes — What Was Wrong and Why

Three bugs were found and fixed in the original codebase. All fixes are in the source files included with this document.

## Bug 1 — NVS Read After Close (net.cpp)

🐛 Severity: High This bug silently discards the user's saved timezone setting and applies the wrong timezone offset on every boot after the first.

### What the original code did:

prefs.begin("clock", false);

int savedTZ = prefs.getInt("tz", TZ_OFFSET_SEC);

// ... (save new value if changed)

prefs.end(); // ← NVS namespace is CLOSED here

// BUG: Reading from a closed NVS handle!

long tz = prefs.getInt("tz", TZ_OFFSET_SEC); // ← always returns the default

configTime(tz, DST_OFFSET_SEC, ...);

### Why it was wrong:

prefs.end() closes the NVS (Non-Volatile Storage) namespace. Any read after this returns the default value, not what's stored in flash. So the timezone saved in the setup portal was never actually applied — it always used the compile-time default TZ_OFFSET_SEC.

### The fix:

prefs.begin("clock", false);

int savedTZ = prefs.getInt("tz", TZ_OFFSET_SEC);

long resolvedTZ = savedTZ; // capture resolved value in a local variable

if (connected) {

String portalVal = String(tzParam.getValue());

if (portalVal.length() > 0) {

long portalTZ = portalVal.toInt();

if (portalTZ != savedTZ) {

prefs.putInt("tz", (int)portalTZ);

resolvedTZ = portalTZ; // use new value immediately

}

}

}

prefs.end(); // ← close AFTER all reads and writes

configTime(resolvedTZ, DST_OFFSET_SEC, ...); // ← uses local variable, not prefs

## Bug 2 — OTA Over HTTP Only (net.cpp)

🐛 Severity: High OTA update checks and firmware downloads silently fail for HTTPS URLs. Since all modern file hosting (GitHub, S3, etc.) uses HTTPS, OTA updates effectively did not work.

### What the original code did:

// version.json fetch — used basic HTTPClient without a secure client

HTTPClient http;

http.begin(OTA_VERSION_URL); // ← fails silently for https:// URLs

// Firmware download — used plain WiFiClient

WiFiClient client; // ← plain TCP only, not SSL

httpUpdate.update(client, url);

### Why it was wrong:

WiFiClient is a plain TCP socket. HTTPS requires TLS encryption — you need WiFiClientSecure instead. Calling http.begin(url) with an https:// URL and a plain client causes an immediate connection failure, which the original code logged but did not alert the user about.

### The fix:

#include <WiFiClientSecure.h> // added to top of net.cpp

// For version.json fetch:

WiFiClientSecure secureClient;

secureClient.setInsecure(); // accept any SSL cert (convenient)

HTTPClient http;

http.begin(secureClient, OTA_VERSION_URL);

// For firmware download:

WiFiClientSecure secureClient;

secureClient.setInsecure();

httpUpdate.update(secureClient, url);

// Note: setInsecure() trusts all certificates — fine for home devices.

// For production, load the server's root CA with:

// secureClient.setCACert(rootCACertString);

## Bug 3 — OTA Check Loop Logic (net.cpp)

🐛 Severity: Medium The OTA check timer had a subtle logic issue that could skip the very first check or trigger it unexpectedly depending on millis() overflow behaviour.

### What the original code did:

if (_lastOTACheck == 0 ¦¦ (now - _lastOTACheck) > OTA_CHECK_INTERVAL) {

_lastOTACheck = now;

checkOTA(); // ← initial OTA check already happens in setup()

} // so loop() was checking immediately on first call too

### The fix:

The initial OTA check is explicitly called in setup() via net.checkOTA(). Inside loop(), the timer is initialised to 0, so the first call to loop() would trigger another immediate OTA check. The fix separates the "first check" flag from the interval timer and ensures the loop's check only triggers after a full interval has passed.

bool firstCheck = (_lastOTACheck == 0);

bool intervalDue = (now - _lastOTACheck) >= OTA_CHECK_INTERVAL;

if (firstCheck ¦¦ intervalDue) {

_lastOTACheck = now;

checkOTA();

}

// Note: The setup() call is kept, so the device checks on boot,

// then again after every OTA_CHECK_INTERVAL (default: 6 hours).

# 9  OTA Updates — How Remote Firmware Updates Work

## What OTA Means

"Over-The-Air" means the device downloads and installs new firmware from the internet — no USB cable required. It's how phones, routers, and smart TVs update themselves. For an ESP32, it works by downloading a new firmware.bin and writing it to a second partition in flash memory.

## The ESP32 OTA Partition Scheme

The 4 MB flash chip is divided into named regions (partitions) by the partitions.bin file:

| Partition | Offset | Size | Purpose |
| --- | --- | --- | --- |
| bootloader | 0x0000 (0) | 4 KB | Runs first. Decides which app partition to boot. |
| partition_table | 0x8000 (32768) | 3 KB | The partitions.bin — map of what's where. |
| boot_app0 | 0xE000 (57344) | 8 KB | OTA selection helper from Espressif. |
| app0 (OTA_0) | 0x10000 (65536) | ~1 MB | Main application slot. Your firmware runs here. |
| app1 (OTA_1) | ~0x110000 | ~1 MB | OTA update slot. New firmware downloaded here. |
| nvs | ~0x210000 | 24 KB | WiFi credentials, timezone, user settings (Preferences). |

When an OTA update runs: the new firmware is written to the inactive slot (e.g., OTA_1 while running from OTA_0). On reboot, the bootloader switches to the new slot. If it fails to boot, it falls back to the old slot automatically.

## Setting Up Your OTA Server

You need to host two files on any public HTTPS web server:

### Step 1 — Create version.json

{

"version": "1.0.1",

"firmware_url": "https://your-server.com/clock/firmware.bin",

"release_notes": "Added fourth clock face"

}

### Step 2 — Host the files (GitHub Releases — free)

# 1. Create a new GitHub Release in your repository

# 2. Attach firmware.bin to the release as an asset

# 3. Get the raw asset URL — it looks like:

# https://github.com/yourname/repo/releases/download/v1.0.1/firmware.bin

# Host version.json separately — GitHub Pages is convenient:

# https://yourname.github.io/repo/version.json

### Step 3 — Update config.h

// In include/config.h:

#define FIRMWARE_VERSION "1.0.1" // bump this for each release

#define OTA_VERSION_URL "https://yourname.github.io/clock/version.json"

## OTA Update Flow Diagram

Every 6 hours (OTA_CHECK_INTERVAL), and once on boot, this happens:

Clock boots / 6 hours pass

│

▼

Fetch OTA_VERSION_URL (version.json) over HTTPS

│

├─ HTTP error → log and return false (try again next interval)

│

▼

Parse JSON: get "version" and "firmware_url"

│

├─ version == FIRMWARE_VERSION → "Up to date" → return false

│

▼

version is DIFFERENT → download firmware_url

│

▼

Show progress bar on OLED (0% → 100%)

│

▼

Write to OTA_1 flash partition

│

▼

Reboot → bootloader switches to new partition → running new firmware

# 10  WiFi Setup Flow — The Captive Portal

## What Is a Captive Portal?

A captive portal is the page that pops up when you connect to an airport or hotel WiFi and have to accept terms before getting internet. WiFiManager creates one on the ESP32 itself, so users can configure the clock's WiFi without any coding.

## First Boot Flow

- Power on the clock. No WiFi credentials are saved yet.

- The clock creates its own WiFi hotspot named "SmartClock-Setup" with password "clocksetup".

- On your phone or laptop, connect to this network.

- A captive portal page opens automatically (or visit 192.168.4.1).

- The page shows a list of nearby WiFi networks. Select yours, enter your password.

- There is also a "Timezone offset (seconds from UTC)" field. Enter your offset (e.g. 19800 for IST). This is saved to flash and survives reboots.

- Click Save. The clock connects to your home WiFi, syncs time, and starts the clock face.

- From now on, the clock reconnects automatically on every boot (credentials are saved in NVS).

## Resetting WiFi (Moving to a New Network)

- Hold the BOOT button for 2+ seconds while the clock is running.

- The screen shows "WiFi reset! Rebooting..." and calls WiFi.disconnect(true, true) which erases saved credentials.

- After reboot, the captive portal hotspot opens again.

## Config Portal Timeout

If nobody connects to the setup portal within 180 seconds (PORTAL_TIMEOUT in config.h), WiFiManager gives up and the clock runs in offline mode — it still shows elapsed boot time as a fallback clock.

# 11  The Display — Three Clock Faces & Rendering

## How OLED Drawing Works

The Adafruit SSD1306 library maintains an in-memory pixel buffer (128 × 64 bits = 1 KB of RAM). When you call drawing functions, they modify this buffer in RAM. Nothing appears on screen until you call:

_oled.display(); // flush RAM buffer → OLED via I²C

This is exactly like the HTML Canvas API — you draw to a context in memory and the display is only updated when you commit.

## The Three Clock Faces

| Face | Key Visual Elements | Technical Notes |
| --- | --- | --- |
| FACE_DIGITAL | Large HH:MM (size 3 font), blinking colon every second, seconds progress bar, date row at bottom, WiFi indicator dot. | Colon blink implemented by drawing a black rectangle over the colon pixels, then two circles as dots. Progress bar uses map() to scale seconds 0-59 to pixels 0-124. |
| FACE_ANALOG | Round clock face with 60 tick marks, 12/3/6/9 numerals, hour/minute/second hands, time and date panel on the right. | Hands use trigonometry: sin/cos to convert angle to x/y pixel coordinates. Hour hand moves smoothly (includes minute offset). Second hand has a counterbalance tail. |
| FACE_MINIMAL | Giant hour (size 4 font), medium minute, tiny seconds, full day name, date, row of 12 progress dots along bottom. | Dot row: 60 seconds / 12 dots = one dot per 5 seconds. Filled circle = elapsed, outline circle = remaining. |

## Trig in the Analog Face (Plain English)

Clock hands point in a direction from the centre. Direction is described as an angle (0° = 12 o'clock, 90° = 3 o'clock, etc.). To turn an angle into pixel coordinates:

// angle in radians (convert degrees with * DEG_TO_RAD)

int tip_x = centre_x + hand_length * sin(angle); // horizontal offset

int tip_y = centre_y - hand_length * cos(angle); // vertical offset (screen Y is inverted)

// Example: minute hand at 15 minutes

float angle = (15 * 60 + 0) / 10.0f * DEG_TO_RAD; // 90 degrees = 3 o'clock position

_oled.drawLine(cx, cy, cx + len*sin(angle), cy - len*cos(angle), WHITE);

## Brightness / Screensaver

The SSD1306 has a hardware contrast register. After SCREENSAVER_MS (default 60 seconds) of no button presses, the firmware calls:

_oled.ssd1306_command(SSD1306_SETCONTRAST);

_oled.ssd1306_command(dim ? low_brightness : normal_brightness);

This dims the screen to near-black without stopping redraws, preventing pixel burn-in on OLED displays that are left on in a static position.

# 12  How to Customise — Your Roadmap to New Features

## Easy Changes (Edit config.h only)

| What to change | Setting in config.h |
| --- | --- |
| Default timezone | TZ_OFFSET_SEC (e.g. 0 for UTC, 3600 for CET) |
| Screen dims after X seconds | SCREENSAVER_MS (set 0 to disable) |
| Hotspot name / password | AP_NAME, AP_PASSWORD |
| OTA server URL | OTA_VERSION_URL |
| How often to check for updates | OTA_CHECK_INTERVAL |
| Clock refresh rate | CLOCK_UPDATE_MS (lower = smoother, more CPU) |
| OLED I²C address | OLED_ADDR (try 0x3D if screen is blank) |

## Adding a New Clock Face

- Add a new value to the ClockFace enum in include/display.h before FACE_COUNT.

- Add a private method void _drawMyFace(struct tm *t); declaration in include/display.h.

- Implement the method in src/display.cpp drawing whatever you want.

- Add a case FACE_MYFACE: _drawMyFace(t); break; inside the update() switch statement in display.cpp.

## Adding a New Config Portal Parameter

- Add a WiFiManagerParameter in net.cpp connect() (follow the timezone example).

- Save the value to NVS with prefs.putString("mykey", value) or prefs.putInt()

- Read it back on boot with prefs.getString("mykey", "default")

## Adding a Sensor (Temperature, Humidity, etc.)

- Wire the sensor to an available GPIO pin. Update config.h with the pin number.

- Add the sensor library to lib_deps in platformio.ini

- Create include/sensor.h and src/sensor.cpp following the pattern of display.h/cpp.

- Call sensor.read() in loop() and pass the value to the display for rendering.

# 13  Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Blank OLED screen on boot | Wrong I²C address or wiring | Check SDA=GPIO20, SCL=GPIO21. Try changing OLED_ADDR to 0x3D in config.h. LED blinks rapidly = OLED not detected. |
| LED blinks rapidly and device freezes | display.begin() returned false | OLED not found — verify wiring. Check that 3.3V is reaching OLED VCC. |
| Wrong time shown (offset hours) | Wrong TZ_OFFSET_SEC | Hold BOOT for 2s to reset WiFi, reconnect via portal and enter correct offset. Or change TZ_OFFSET_SEC in config.h. |
| Clock says "Syncing time..." forever | NTP blocked or no internet | Check internet connection. Try pinging pool.ntp.org from another device on same network. |
| "SmartClock-Setup" hotspot not appearing | Saved WiFi credentials are failing | Hold BOOT for 2s to force portal. Or delete .pio and reflash. |
| OTA check fails (HTTP error in serial log) | OTA_VERSION_URL placeholder not replaced | Edit config.h and set a real HTTPS URL to your version.json. Until then, OTA will silently fail every 6 hours. |
| Build fails with "library not found" | PlatformIO cache issue | Run pio pkg install then pio run again. Or rm -rf .pio and rebuild. |
| Upload fails / device not found | Wrong USB port or device in wrong mode | Run pio device list to find the port. Hold BOOT, tap RESET, release BOOT to enter upload mode. |
| Crash / reboot loop (Guru Meditation Error) | Null pointer, stack overflow, or watchdog | Open serial monitor immediately after reset. The error decoder will show which line crashed. |
| "Wrong time after WiFi reset" | Bug 1 (now fixed) — prefs read after close | Use the fixed net.cpp from this document. The timezone is now properly saved and read. |
| "OTA never downloads anything" | Bug 2 (now fixed) — plain HTTP only | Use the fixed net.cpp. WiFiClientSecure is now used for both the version check and download. |

## Reading the Serial Monitor Output

Every module prefixes its log lines with a tag in square brackets. This lets you quickly find where a problem is occurring:

| Log prefix | Source module | Example |
| --- | --- | --- |
| [MAIN] | main.cpp | [MAIN] Setup complete. Entering main loop. |
| [NET] | net.cpp | [NET] Connected! IP: 192.168.1.42 |
| [OTA] | net.cpp (OTA section) | [OTA] Checking for update at: https://... |
| [ERROR] | main.cpp (fatal) | [ERROR] OLED not found. Check SDA/SCL wiring. |

End of Document

ESP32-C3 Smart Clock — Full Technical Reference — Bug-Fixed Edition